|  | Life Support Team Mission Day Instructions |
| --- | --- |

Overview

The Environmental Control and Life Support (ECLS) system ensures that atmospheric gases stay within a normal range and are free of contaminants. A disruption to any part of the ECLS system could be dangerous.

Using the Internet, your team will be able to download real-time readings from an instrument called the major constituent analyzer. You must graph the data and make calculations. Communicate with the other teams and recommend ways to protect the space station crew.

Mission Day Materials

One computer for real-time data

Life support reference guide

15 blank report forms on colored paper (blue would be best) to deliver to the communications team

Calculators

Mission day instructions

Life support data graphs and instructions

Life support data tracking tables

###### Your Task

By the time the mission starts, the life support team should be able to:

- Download real-time data, record it, graph it, and make calculations.

- Monitor changes in the atmosphere of the station and report the possible effects on the astronauts.

- Use the life support reference guide to understand hypoxia and carbon dioxide poisoning.

- Recommend options and adjustments to avoid a dangerous situation.

Team Tasks

These tasks are listed in priority order. Next to each task, assign a team member. Depending on the size of your team, you may need to assign two tasks to one person.

|  | Team leader: Makes sure the team is working quickly and efficiently. Determines whether there are any concerns and helps the team decide what to do about them. |
| --- | --- |
|  | Data graphing: Records real-time data on graphs and makes predictions. (May be combined with data analysis tasks if necessary.) |
|  | Data analysis (O2): Records real-time data in data tracking tables and conducts analyses. Completes report forms every few minutes. |
|  | Data analysis (CO2): Records real-time data in data tracking tables and conducts analyses. Completes report forms every few minutes. |
|  | Crisis management data runner: Completes report forms every few minutes. Writes down questions from mission control and responds. (May be combined with team leader tasks if necessary.) |
|  | Data recording: Records real-time data from the computer. The data will be accessed and downloaded after the start of the mission. (May be combined with other tasks if necessary.) |
|  | Research and reference: Reads and understands information provided in the reference guide to make recommendations. (May be combined with other tasks if necessary.) |
|  | Reporter/Graphic organizer: Takes notes during the mission like a reporter from a newspaper or TV station. Writes down all the emergencies, recommendations, choices, and successes. May want to use a white board or chalkboard posted on the wall to inform other teams about what is going on with your team. |

|  | Life Support Team Graphs and Instructions |
| --- | --- |

Instructions

You need to create graphs of the O2 and CO2 levels. Use the data from column C on each data tracking table for the y-axis values and plot them along the x-axis by the time of day. This graph will help determine whether or not the changes are a concern. Examine the slope of the line between the data points for the current reading and the last reading. You may want to use a ruler to do this. Is the slope of the line showing a trend moving toward or away from a danger area?

| Oxygen Levels ► |  |
| --- | --- |
| Carbon Dioxide Levels ► |  |

|  | Life Support Team Data Analysis Instructions |
| --- | --- |

| You will receive pressure readings every few minutes from the major constituent analyzer. Analyze them quickly. Important! Before mission day use the practice worksheets and practice graphs to prepare yourself. The instructions below explain how to complete the data tracking table on the next page. Find the sheet labeled “Life Support Team Data Tracking Table.” Make one copy for oxygen and one for carbon dioxide. |  |
| --- | --- |
| Column A: Time of Day This is like military time. Time is given using a 24-hour clock with no “a.m.” or “p.m.” For example, one o’clock in the morning is 01:00, and 4:20 in the afternoon is 16:20, while 11:15 at night is 23:15. |  |

Column B: Real-time Data

Record the current reading from the data for the gas you are tracking (O2, or CO2) in column B.

You will receive data for each component as a percentage, for example O2 = 25 percent. This indicates what percentage of the air inside the station is made up of oxygen.

Column C: Conversion to Pressure

Column C = 760 mmHg X Column B

divided by 100

We need to know the pressure to know how safe or dangerous the situation is. To convert from percent to mmHg, multiply the total pressure of 760 mmHg by the current reading from column B and divide by 100.

Your result will be in mmHg, which is millimeters of mercury. This is the common unit that scientists use for atmospheric gases.

Column D: Change in Pressure

Column D =

Current reading from column C minus the previous reading from column C

How much are the readings changing? To determine this, subtract the previous reading in column C from the current reading in column C. Your result will be in mmHg.

Column E: Rate of Change

Column E = Column D (mmHg)

Time elapsed (hours)

The rate of change tells us how quickly things are changing. Take the result from column D and divide by the amount of time. There have been 0.33 hours since the last reading. This gives a rate of change in mmHg per hour.

Column F: Trend

Is the dose increasing or decreasing?

Write down the word “increasing” or “decreasing” to describe whether the levels are higher or lower than the last reading. If they are higher from one reading to the next, then the rate is “increasing.”

To determine whether or not these changes are a concern, use the line graph. Examine the slope of the line between the data points for the current reading and the last reading. You may want to use a ruler to do this. Is the slope of the line showing a trend moving toward or away from a danger area?

|  | Life Support Team Data Tracking Table |  |
| --- | --- | --- |

Examples and Practice

Circle One: 		Oxygen (O2) 	Carbon Dioxide (CO2)

| Column | A | B | C (Graph this column) | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Table Headings | Time of Day | Content of O2 or CO2 | Pressure of O2 or CO2 | Change in O2 or CO2 | Rate | Trend |
| Units | 24-hour Clock | Percent (%) | mmHg | mmHg | mmHg per hour |  |
| Calculations | From Data | From Data | C= 760 x B/100 | D = Current C - Previous C | E = D/0.33 | # Look at the graph and check one: |
| Examples and Practice | 15:00 | (This example shows data for CO2) 0.35 | 2.66 | n/a | n/a | □ Increasing. □ Decreasing. |
|  | 15:20 | 0.57 | 4.33 | 1.67 | 5.06 | X Increasing. □ Decreasing. |
|  | 15:40 | 0.37 | 2.81 | -1.52 | -4.61 | □ Increasing. X Decreasing. |
|  | 16:00 | 0.80 | 6.08 | 3.27 | 9.91 | X Increasing. □ Decreasing. |
|  | 16:20 | 1.20 | 9.12 | 3.04 | 9.21 | X Increasing. □ Decreasing. |
|  | 16:40 | 1.40 |  |  |  | □ Increasing. □ Decreasing. |
|  | 17:00 | 0.90 |  |  |  | □ Increasing. □ Decreasing. |
|  | 17:20 | 1.80 |  |  |  | □ Increasing. □ Decreasing. |
|  | 17:40 | 0.95 |  |  |  | □ Increasing. □ Decreasing. |
|  | 18:00 | 0.74 |  |  |  | □ Increasing. □ Decreasing. |
|  | 18:20 | 0.51 |  |  |  | □ Increasing. □ Decreasing. |

Note: Round all calculations to two decimal places.

For additional practice use these values for O2: You will need to print out the next page.

| Time | Content of O2 |
| --- | --- |
| 16:40 | 20.15 |
| 17:00 | 19.50 |
| 17:20 | 18.65 |
| 17:40 | 21.33 |
| 18:00 | 21.56 |
| 18:20 | 21.60 |

|  | Life Support Team Data Tracking Table |  |
| --- | --- | --- |

For the Mission

Circle One: 		Oxygen (O2) 	Carbon Dioxide (CO2)

| Column | A | B | C (Graph this column) | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Table Headings | Time of Day | Content of O2 or CO2 | Pressure of O2 or CO2 | Change in O2 or CO2 | Rate | Trend |
| Units | 24-hour Clock | Percent (%) | mmHg | mmHg | mmHg per hour |  |
| Calculations | From Data | From Data | C= 760 x B/100 | D = Current C - Previous C | E = D/0.33 | # Look at the graph and check one |
| For the Mission | 15:00 |  |  | n/a | n/a | □ Increasing. □ Decreasing. |
|  | 15:20 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 15:40 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 16:00 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 16:20 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 16:40 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 17:00 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 17:20 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 17:40 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 18:00 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 18:20 |  |  |  |  | □ Increasing. □ Decreasing. |
|  | 18:40 |  |  |  |  | □ Increasing. □ Decreasing. |

Note: Round all calculations to two decimal places.