Life Support Team

Data Tracking Table

Examples and Practice

Circle One:			Oxygen (O2)			Carbon Dioxide (CO2)

| Column | A | B | C (Graph this column) | D | E | F |  | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table Headings | UTC | Content of O2 or CO2 | Partial Pressure of O2 or CO2 | Change in O2 or CO2 | Rate | Direction of Change |  | Time to Criticality** |
| Units | 24-hour Clock | % | mmHg | mmHg | mmHg per hour |  |  | hours |
| Calculations | From Data | From Data | C= 760 x B/100 | D= Current C – Previous C | E= D/0.33 | Look at the graph and check one |  | G= Critical Value-C E |
| Examples and Practice | 15:00 | 0.35 | 2.66 | n/a | n/a | n/a |  | n/a |
|  | 15:20 | 0.57 | 4.33 | 1.67 | 5.07 | toward |  | 0.64 |
|  | 15:40 | 0.37 | 2.81 | -1.52 | -4.61 | away |  | n/a |
|  | 16:00 | 0.80 | 6.08 | 3.27 | 9.91 | toward |  | 0.15 |
|  | 16:20 | 1.20 | 9.12 | 3.04 | 9.21 | critical |  | n/a |
|  | 16:40 | 1.40 | 10.64 | 1.52 | 4.61 | critical |  | n/a |
|  | 17:00 | 0.90 | 6.84 | -3.80 | -11.52 | away |  | n/a |
|  | 17:20 | 1.80 | 13.68 | 6.84 | 20.73 | critical |  | n/a |
|  | 17:40 | 0.95 | 7.22 | -6.46 | -19.58 | away |  | n/a |
|  | 18:00 | 0.74 | 5.62 | -1.60 | -4.85 | away |  | n/a |
|  | 18:20 | 0.51 | 3.88 | -1.74 | -5.27 | away |  | n/a |

Note: Round all calculations to two decimal places. Critical Value for O2 is 115mmHg. Critical Value for CO2 is 7.6 mmHg.

Life Support Team

Data Tracking Table

Examples and Practice

Circle One:			Oxygen (O2)			Carbon Dioxide (CO2)

| Column | A | B | C (Graph this column) | D | E | F |  | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table Headings | UTC | Content of O2 or CO2 | Partial Pressure of O2 or CO2 | Change in O2 or CO2 | Rate | Direction of Change |  | Time to Criticality** |
| Units | 24-hour Clock | % | mmHg | mmHg | mmHg per hour |  |  | hours |
| Calculations | From Data | From Data | C= 760 x B/100 | D= Current C – Previous C | E= D/0.33 | Look at the graph and check one |  | G= Critical Value-C E |
| Examples and Practice | 15:00 | 21.70 | 164.92 | n/a | n/a | n/a |  | n/a |
|  | 15:20 | 21.42 | 162.79 | -2.13 | -6.45 | toward |  | 7.41 |
|  | 15:40 | 21.75 | 165.30 | 2.51 | 7.61 | away |  | n/a |
|  | 16:00 | 19.80 | 150.48 | -14.82 | -44.91 | toward |  | 0.79 |
|  | 16:20 | 18.45 | 140.22 | -10.26 | -31.09 | toward |  | 0.81 |
|  | 16:40 | 20.15 | 153.14 | 12.92 | 39.15 | away |  | n/a |
|  | 17:00 | 19.50 | 148.20 | -4.94 | -14.97 | toward |  | 2.22 |
|  | 17:20 | 18.65 | 141.74 | -6.46 | -19.58 | toward |  | 1.37 |
|  | 17:40 | 21.33 | 162.11 | 20.37 | 61.73 | away |  | n/a |
|  | 18:00 | 21.56 | 163.86 | 1.75 | 5.30 | away |  | n/a |
|  | 18:20 | 21.60 | 164.16 | 0.30 | 0.91 | away |  | n/a |

Note: Round all calculations to two decimal places. Critical Value for O2 is 115mmHg. Critical Value for CO2 is 7.6 mmHg.

Practice Data

Radiation Team

Data Tracking Table

Circle One:			TEPC1 (Portable)			TEPC2 (Stationary)

| Column | A | B | C (Graph this column) | D | E | F (Graph this column and compare to tables in the Reference Guide) |
| --- | --- | --- | --- | --- | --- | --- |
| Table Headings | UTC | 20 min Dose Total | Cumulative Dose | Dose Rate | Time to Criticality | 24 hour Projected Total |
| Units | 24 Hour Clock | rems | rems | rem/hr | Hours | rems |
| Calculations | From Data | From Data | C= B + Previous C | D= B . 0.33 | E= 100 – C D | F= (D x 24) + C |
| Examples and Practice | 15:00 | 0.43 | 0.43 | 1.30 | 76.59 | 31.63 |
|  | 15:20 | 0.58 | 1.01 | 1.76 | 56.24 | 43.25 |
|  | 15:40 | 1.67 | 2.68 | 5.06 | 19.23 | 124.12 |
|  | 16:00 | 3.56 | 6.24 | 10.79 | 8.69 | 265.20 |
|  | 16:20 | 5.01 | 11.25 | 15.18 | 5.85 | 375.57 |
|  | 16:40 | 4.20 | 15.45 | 12.73 | 6.64 | 320.97 |
|  | 17:00 | 1.83 | 17.28 | 5.55 | 14.90 | 150.48 |

Note: Round all calculations to two decimal places.

Circle One:			TEPC1 (Portable)			TEPC2 (Stationary)

| Column | A | B | C (Graph this column) | D | E | F (Graph this column and compare to tables in the Reference Guide) |
| --- | --- | --- | --- | --- | --- | --- |
| Table Headings | UTC | 20 min Dose Total | Cumulative Dose | Dose Rate | Time to Criticality | 24 hour Projected Total |
| Units | 24 Hour Clock | rems | rems | rem/hr | Hours | rems |
| Calculations | From Data | From Data | C= B + Previous C | D= B . 0.33 | E= 100 – C D | F= (D x 24) + C |
| Examples and Practice | 15:00 | 0.43 | 0.43 | 1.30 | 76.59 | 31.63 |
|  | 15:20 | 0.58 | 1.01 | 1.76 | 56.24 | 43.25 |
|  | 15:40 | 1.67 | 2.68 | 5.06 | 19.23 | 124.12 |
|  | 16:00 | 3.56 | 6.24 | 10.79 | 8.69 | 265.20 |
|  | 16:20 | 5.01 | 11.25 | 15.18 | 5.85 | 375.57 |
|  | 16:40 | 4.20 | 15.45 | 12.73 | 6.64 | 320.97 |
|  | 17:00 | 1.83 | 17.28 | 5.55 | 14.90 | 150.48 |

Note: Round all calculations to two decimal places.

Practice TEPC 1 & 2

STORM Team

Data Tracking Table: X-Ray Production

| Column | A | B (Graph this column) | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table Headings | UTC | X-Rays | Category | Change | Rate | Projected X-Ray Production in 1 Hour | Category |
| Units | 24-hour Clock | Amount of X-Rays | From R1 to R5 | Amount of X-Rays | X-Ray/min | X-Ray Production | From R1 to R5 |
| Calculations | From Data | From Data | See Reference Guide | B – Previous B | D/20 min | (E x 60 min) + B | See Reference Guide |
| Examples and Practice | 15:00 | 9 | R1 | n/a | n/a | n/a | n/a |
|  | 15:20 | 16 | R1 | 7 | .35 | 37 | R1 |
|  | 15:40 | 46 | R1 | 30 | 1.5 | 136 | R3 |
|  | 16:00 | 1,250 | R4 | 1,204 | 60.2 | 4,850 | R5 |
|  | 16:20 | 90 | R2 | -1,160 | -58 | 0 | n/a |
|  | 16:40 | 452 | R3 | 362 | 18.1 | 1,538 | R4 |
|  | 17:00 | 892 | R3 | 440 | 22 | 2,212 | R5 |
|  | 17:20 | 1,029 | R4 | 137 | 6.85 | 1,440 | R4 |
|  | 17:40 | 1,876 | R4 | 847 | 42.35 | 4,417 | R5 |
|  | 18:00 | 384 | R3 | -1,492 | -74.6 | 0 | n/a |
|  | 18:20 | 236 | R3 | -148 | -7.4 | 0 | n/a |

STORM Team

Data Tracking Table: Proton Production

| Column | A | B (Graph this column) | C | D | E | F (Use graph or charts) | G |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table Headings | UTC | Protons | Category | Change | Rate | Projected 24 Hour Proton Production | Category |
| Units | 24-hour Clock | Amount of Protons | From S1 to S5 | Amount of Protons | Proton/min | Amount of Protons | From S1 to S5 |
| Calculations | From Data | From Data | See Reference Guide | B – Previous B | D/20 min | (E x 1440 min) + B | See Reference Guide |
| Examples and Practice | 15:00 | 45 | S1 | n/a | n/a | n/a | n/a |
|  | 15:20 | 119 | S2 | 74 | 3.7 | 5,447 | S3 |
|  | 15:40 | 1,310 | S3 | 1,191 | 59.55 | 87,062 | S4 |
|  | 16:00 | 350 | S2 | -960 | -48 | 0 | n/a |
|  | 16:20 | 765 | S2 | 415 | 20.75 | 29,880 | S4 |
|  | 16:40 | 1,890 | S3 | 1,125 | 56.25 | 82,890 | S4 |
|  | 17:00 | 3,410 | S3 | 1,520 | 76 | 112,850 | S5 |
|  | 17:20 | 2,525 | S3 | -885 | -44.25 | 0 | n/a |
|  | 17:40 | 4,100 | S3 | 1,575 | 78.75 | 117,500 | S5 |
|  | 18:00 | 3,575 | S3 | -525 | -26.25 | 0 | n/a |
|  | 18:20 | 780 | S2 | -2,795 | -139.75 | 0 | n/a |

| X-Ray Amount (microwatts) | X-Ray Category | Descriptor |
| --- | --- | --- |
| >2000 | R5 | Extreme |
| 1000 - 2000 | R4 | Severe |
| 100 – 1000 | R3 | Strong |
| 50 – 100 | R2 | Moderate |
| 0 – 50 | R1 | Minor |

X-Ray Practice

Proton Practice